CVA6 Cold RAM-to-Register Benchmark Results (1 Iteration + Reset)

Payload: 4,096 Bytes | Frequency: 50 MHz | Hardware Reset between Runs

| Algorithm | Block size (bits) | Digest size (bits) | Datapath size (bits) | Clk. Cycles | Instructions | Time (s) @ 50MHz |
| --- | --- | --- | --- | --- | --- | --- |
| SHA2-224 | 512 | 224 | 32 | 280,045 | 247,142 | 0.005601 |
| SHA2-256 | 512 | 256 | 32 | 279,851 | 247,133 | 0.005597 |
| SHA2-384 | 1024 | 384 | 64 | 204,596 | 165,733 | 0.004092 |
| SHA2-512 | 1024 | 512 | 64 | 204,665 | 165,791 | 0.004093 |
| SHA2-512/224 | 1024 | 224 | 64 | 204,847 | 165,790 | 0.004097 |
| SHA2-512/256 | 1024 | 256 | 64 | 204,721 | 165,771 | 0.004094 |
| SHA3-224 | 1152 | 224 | 64 | 252,879 | 203,677 | 0.005058 |
| SHA3-256 | 1088 | 256 | 64 | 267,902 | 215,798 | 0.005358 |
| SHA3-384 | 832 | 384 | 64 | 340,875 | 277,631 | 0.006817 |
| SHA3-512 | 576 | 512 | 64 | 479,850 | 394,522 | 0.009597 |
| SHAKE-128 | 1344 | 256 | 64 | 220,692 | 176,161 | 0.004414 |
| SHAKE-256 | 1088 | 512 | 64 | 268,146 | 215,837 | 0.005363 |

Table 2: Comparison — Warm Cache vs. Cold RAM Load (1 Iteration + Reset)

| Algorithm | Warm Cycles | Cold Cycles (1 Iter) | RAM Load Overhead | Warm Time (s) | Cold Time (s) |
| --- | --- | --- | --- | --- | --- |
| SHA2-224 | 253,888 | 280,045 | +10.30% (+26,157 cyc) | 0.005079 | 0.005601 |
| SHA2-256 | 253,920 | 279,851 | +10.21% (+25,931 cyc) | 0.005078 | 0.005597 |
| SHA2-384 | 177,633 | 204,596 | +15.18% (+26,963 cyc) | 0.003553 | 0.004092 |
| SHA2-512 | 177,659 | 204,665 | +15.20% (+27,006 cyc) | 0.003554 | 0.004093 |
| SHA2-512/224 | 177,635 | 204,847 | +15.32% (+27,212 cyc) | 0.003554 | 0.004097 |
| SHA2-512/256 | 177,635 | 204,721 | +15.25% (+27,086 cyc) | 0.003553 | 0.004094 |
| SHA3-224 | 246,293 | 252,879 | +2.67% (+6,586 cyc) | 0.004926 | 0.005058 |
| SHA3-256 | 262,551 | 267,902 | +2.04% (+5,351 cyc) | 0.005251 | 0.005358 |
| SHA3-384 | 336,223 | 340,875 | +1.38% (+4,652 cyc) | 0.006725 | 0.006817 |
| SHA3-512 | 475,319 | 479,850 | +0.95% (+4,531 cyc) | 0.009507 | 0.009597 |
| SHAKE-128 | 213,623 | 220,692 | +3.31% (+7,069 cyc) | 0.004273 | 0.004414 |
| SHAKE-256 | 262,663 | 268,146 | +2.09% (+5,483 cyc) | 0.005254 | 0.005363 |

Architectural Note: Every single algorithm executed exactly 29 fewer retired instructions compared to the multi-algorithm loop build (due to compiler loop/switch pruning in the single-target build). The +26k clock cycle increase in SHA-2 represents the pure hardware latency of cold L1 cache misses fetching instructions and the 4 KB test payload across the AXI bus from DRAM/SRAM into CPU registers.